## ROS Bag Motion Metrics Report

### Processing Notes

- Metrics are computed only inside the evaluation windows from Vicon Cutoff+5s until Landing.

- Horizontal velocity is cleaned before peak/RMS calculation using x/y component bounds of -8 to +8 m/s, then computed from cleaned x/y components.

- X and Y velocity component plots use cleaned x/y values directly.

- Horizontal acceleration is cleaned before peak/RMS calculation using x/y component bounds of -8 to +8 m/s^2, then computed from cleaned x/y components.

- Savitzky-Golay smoothing is disabled for cleaned velocity and acceleration so peak metrics are not smoothed down.

- Angular rate magnitude uses raw x/y/z components.

- Roll and pitch are converted from ROS quaternion fields x/y/z/w to degrees.

### Formulas

- Horizontal velocity magnitude: sqrt(vx^2 + vy^2)

- Peak horizontal velocity: max(sqrt(vx^2 + vy^2))

- RMS horizontal velocity: sqrt(mean(vx^2 + vy^2))

- Peak absolute X velocity: max(abs(vx))

- Peak absolute Y velocity: max(abs(vy))

- Horizontal acceleration magnitude: sqrt(ax^2 + ay^2)

- Peak acceleration magnitude: max(sqrt(ax^2 + ay^2))

- RMS acceleration magnitude: sqrt(mean(ax^2 + ay^2))

- Angular rate magnitude: sqrt(wx^2 + wy^2 + wz^2)

- Peak angular rate magnitude: max(sqrt(wx^2 + wy^2 + wz^2))

- RMS angular rate magnitude: sqrt(mean(wx^2 + wy^2 + wz^2))

- Peak absolute roll angle: max(abs(roll))

- Peak absolute pitch angle: max(abs(pitch))

### Highlighted New Plots

- RMS horizontal velocity

- Peak absolute roll angle

- Peak absolute pitch angle

### UN

- Flights processed: 8

#### Metric 1: Peak horizontal velocity

#### Metric 2: RMS horizontal velocity - NEW

#### Metric 3: Peak absolute X velocity

#### Metric 4: Peak absolute Y velocity

#### Metric 5: Peak acceleration magnitude

#### Metric 6: RMS acceleration magnitude

#### Metric 7: Peak angular rate magnitude

#### Metric 8: RMS angular rate magnitude

#### Metric 9: Minimum roll angle

#### Metric 10: Maximum roll angle

#### Metric 11: Peak absolute roll angle - NEW

#### Metric 12: Minimum pitch angle

#### Metric 13: Maximum pitch angle

#### Metric 14: Peak absolute pitch angle - NEW

### RAW

- Flights processed: 10

#### Metric 1: Peak horizontal velocity

#### Metric 2: RMS horizontal velocity - NEW

#### Metric 3: Peak absolute X velocity

#### Metric 4: Peak absolute Y velocity

#### Metric 5: Peak acceleration magnitude

#### Metric 6: RMS acceleration magnitude

#### Metric 7: Peak angular rate magnitude

#### Metric 8: RMS angular rate magnitude

#### Metric 9: Minimum roll angle

#### Metric 10: Maximum roll angle

#### Metric 11: Peak absolute roll angle - NEW

#### Metric 12: Minimum pitch angle

#### Metric 13: Maximum pitch angle

#### Metric 14: Peak absolute pitch angle - NEW